- Appendix II

- Key features of the RoboCup NAO 4

#### New ATOM head:

- Intel Processor:

  - Intel® Atom™ Processor Z530

  - (512K Cache, 1.60 GHz, 533 MHz FSB)

  - HyperThreading

  - Instruction Set: 32-bit

  - Instruction Set Extensions: SSE2, SSE3, SSSE3

- 1GB of DDR2 RAM

- 2GB of SATA Flash

- Minimum of 8 Go micro-SD Flash

- 2x HD Cameras (1280x960)

|  | V3.3 | NAO 4 |
| --- | --- | --- |
| Pixel size | 3.6µm x 3.6µm | 1.9µm x 1.9µm |
| Resolution & fps | Up to VGA@30fps | Embedded frame rates: 1280x960@30fps 640x480@30fps |
| Sensitivity per pixel | 1.3V/(Lux.sec) | 2.24V/(Lux.sec) |

  - Features:

    - Digital or analog binning

    - 4x more sensitivity than v3.3

Figure 1: More sensitivity

    - Camera Diagonal Field of View: increased and overlapping

Figure 2: Increased and overlapping Field of View

    - Dedicated and non-programmable FPGA: 2 simultaneous video streams

    - Rolling shutter issues divided by 2.5

    - Low noise levels

- Head thermal management:

  - New Fan: 2x more airflow

  - Airflow control redesign

Figure 3: Temperature Controlled Fan with laminar forced convection

- New Skull/brain paradigm

  - Shock absorbers

  - New connectors with high misalignment tolerances

Figure 4: Shock absorbers (red arrow)

#### Body improvements:

- New spherical joints:

  - Better force distribution

  - Better elasticity coefficient

Figure 5: New spherical joints

- New motor boards (torso and legs) to improve motor thermal management: this improvement is in testing process and therefore subject to approval by the qualification team.

- Lighter robot due to new motor Better motor integration